DC-T4517WRX-A 4.3mm/6.2mm

Architectural and Engineering Specifications

Version 1.0

(Apr. 18, 2023)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-T4517WRX is a bullet type Network Camera (IP Camera) designed and manufactured by IDIS. This camera provides 5M (2592 x 1944) resolution at 30 ips (images per second) with H.265, H.264, and MJPEG compression. This camera is equipped with Fixed-focal lens(f=4.3mm, f=6.2mm), True Day/Night, WDR, PoE (IEEE 802.3af Class 3), IR LED, Vandal-proof bullet enclosure design and IP66 rated.

General Specification

- The IP camera shall be equipped with 5 Megapixel 1/2.8” CMOS Sensor.

- The IP camera shall be equipped with

  - f=4.3mm Fixed-focal, F1.6

  - f=6.2mm Fixed-focal, F1.6

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance. The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have Wide Dynamic Range compensation for improved video quality in high-contrast situations (120 dB).

- The IP camera shall be equipped with 2ea Infrared LED with range up to 20m (65.6ft).

- The IP camera shall utilize 2DNR/3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- IP camera shall be equipped with 10/100 Base-T, auto-sensing, half/full duplex, RJ45 Ethernet connection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3af, Class 3 to supply power to the camera over the network.

- The IP camera shall have video out feature (NTSC/PAL).

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 2592x1944 at rates up to 30ips (images per second).

- The IP camera shall provide direct network connection using H.265, H.264 and MJPEG compression

- The IP camera shall support Quadruple Streams.

- The IP camera shall conform to the ONVIF Profile S.

- The IP camera shall be equipped with embedded web server (IDIS Web) which works independently using a Web Browser with ActivX plug-in.

- The IP camera shall have SSL Encryption, Multi-user Authority, IEEE 802.1x, IP Filtering, HTTPS for greater security.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analysis (VA): Video Motion Detection, Active Tampering Alarm, Trip Zone.

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and M-JPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/2.8" CMOS

- Maximum Resolution: 2592 x 1944

- Scanning Mode: Progressive Scan

- Lens Type: Fixed-focal

  - f=4.3mm, F1.6

  - f=6.2mm, F1.6

- IRIS Control: Fixed Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - f=4.3mm: 80º(H) 56.7º(V) 106.9º(D)

  - f=6.2mm: 49.9º(H) 36.7º(V) 64º(D)

- Minimum Illumination:

  - f=4.3mm > COLOR : 0.15 lux @ F1.6 B/W : 0 lux  (IR LED ON)

  - f=6.2mm > COLOR : 0.15 lux @ F1.6 B/W : 0 lux  (IR LED ON)

- S/N Ratio: more than 45dB

- Maximum Frame Rate:

  - 4:3 mode : 30ips : 2592 x 1944(WDR)

  - 16:9 mode : 30ips : 2592 x 1456(WDR)

- Video Resolution:

  - 4:3 mode : 2592x1944, 1920x1440, 1280x960x 640x480

  - 16:9 mode : 2592x1456, 1920x1080, 1280x720, 640x360

- Video Compression : H.265, H.264, MJPEG

- Video Compression Level: 4 levels - Basic, Standard, High, Very High

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB (True WDR)

- True Day & Night: Yes (ICR)

- IR Distance (The number of LEDs, IR wavelength): 20m / 65.6 ft (2ea)

- Intelligent Video Analytic: Video Motion Detection, Active Tampering Alarm, Trip Zone

Audio Specification

- None

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols: DirectIP 2.0 Protocol, IPv4, IPv6, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP,  RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, Upnp, SNMPv2

- Streaming Mode: Quadruple streaming

Security Specification

- SSL Encryption, Multi-user Authority, IEEE 802.1x, IP Filtering, HTTPS

- Maximum User Access: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

Event Specification

- Trigger Events: Motion Detection, Alarm in, Tampering, Trip Zone

- Event Notification: Remote S/W, Email (with Image)

Environmental Specification

- Operating Temperature: -20°C ~ +55°C (-4°F ~ +131°F)

  - Starting up at above 0°C (32°F)

- Operating Humidity: 0% to 90%

- Vandal-proof Enclosure: Yes (IK10)

- Outdoor Ready: Yes (IP66)

Electrical Specification

- Power Source: PoE (IEEE 802.3af Class 3)

- Power Consumption: 6.5W, PoE (IEEE 802.3af Class 3).

- Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (W x H): 70mm x 65mm x 197.5mm (2.76" x 2.56" x 7.78")

- Unit Weight: 0.4kg (0.88 lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | Global Technical Team | Jul. 29, 2021 | Initial Release |