DC-U6K31WRA

Architectural and Engineering Specifications

Version 1.0.0

(Aug. 25, 2025)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

The DC-U6K31WRA is a 5MP AI multi-directional(4ch) camera. It uses a 1/2.8" CMOS sensor and supports H.265, H.264, and MJPEG compression with IDIS Intelligent Codec. It features a motorized vari-focal lens (3.3–7.02mm, F1.8–F2.74) with remote focus and zoom control. The camera provides True WDR 120dB and IR LED support up to 15m. The housing is rated IK10 vandal-resistant and IP67/NEMA4X weatherproof. AI functions include object detection, intrusion, loitering, line crossing, face detection, and metadata for human, vehicle, and face. The camera supports Smart Failover, using microSD/SDHC/SDXC cards up to 512GB for edge recording during network loss. It is ONVIF compatible (Profile S/T/M [TBD]) and integrates with IDIS DirectIP NVRs. Additional features include alarm I/O (1 in / 1 out), two-way audio, quadruple streaming, and PoE (IEEE 802.3at Class 4, 25W).

General Specification (Multi-directional 4CH)

- The IP camera shall be equipped with a 5 Megapixel 1/2.8” CMOS sensor (IMX675).

- The IP camera shall be equipped with a 3.3 ~ 7.02 mm motorized vari-focal lens, F1.4~F2.74.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120 dB).

- The IP camera shall be equipped with 6ea Infrared LED with range up to 15m.

- The IP camera shall support fixed-Iris.

- The IP camera shall utilize 2DNR/3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The camera shall be enclosed in a dome housing with vandal resistance (IK10) and IP66/NEMA4X weatherproof protection.

- The camera shall have a 10/100/1000Base-T RJ-45 Ethernet port with auto-sensing and full/half-duplex mode.

- The camera shall support Power over Ethernet (PoE) IEEE 802.3at Class 4 and 25W [TBD] power consumption..

- The camera shall include a CVBS video output for local setup and testing.

- The IP camera shall be equipped with an on-board microSD/SDHC/SDXC card slot, supporting up to 512GB for edge recording.

- The camera shall support Smart Failover to prevent data loss during short network interruptions or recorder overload.

- The camera shall support a maximum resolution of 2592x1944 at up to 30 ips (images per second).

- The IP camera shall provide network connection using H.265, H.264, and MJPEG compression.

- The IP camera shall support quadruple streams in DirectIP 2.0 protocol mode.

- The IP camera shall conform to the ONVIF Profile S/T/M.

- The IP camera shall be equipped with an embedded web server (IDIS Web) which works independently using a web browser with ActiveX plug-in.

- The IP camera shall include security features such as: Digest authentication, Password protection, Brute force delay protection, User access log, IP filtering, IEEE802.1x(EAP-TLS), SSL Encryption, HTTPS Encryption.

- The IP camera shall support bandwidth limitation and MAT for optimized network usage.

- The IP camera shall have easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Intelligent Video Analytics, including:

- Object Metadata: Human, Vehicle, Face

- IDLA: Object Detection, Intrusion, Loitering, Line Crossing, Face Detection

- ETC: Video Motion Detection, Active Tampering Alarm

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such as IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/2.8” CMOS

- Maximum Resolution: 2592 x 1944

- Scanning Mode: Progressive Scan

- Lens Type: Motorized Vari-focal (f=3.3 ~ 7.02mm, F1.8 ~ F2.74)

- Iris Control: Fixed Iris

- Angular Field of View (H: Horizontal, V: Vertical, D: Diagonal):

- Wide: 96º(H), 69.8º(V), 127.8º(D)

- Tele: 42.4º(H), 31.8º(V), 53.2º(D)

- Minimum Illumination:

- COLOR: 0.1 lux @ F1.8

- B/W: 0 lux  (IR LED ON)

- S/N Ratio: More than 45 dB

- Maximum Frame Rate:

4:3 mode : 30ips : 2592 x 1944(WDR)

16:9 mode : 30ips : 2592 x 1456(WDR)

- Video Resolution:

4:3 : 2592x1944, 1920x1440, 1280x960, 640x480

16:9 : 2592x1456, 1920x1080, 1280x720, 640x360

- Video Compression: H.265, H.264, MJPEG

- Video Compression Level: 4 levels: Basic, Standard, High, Very High

- Intelligent Codec is supported.

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB(True WDR)

- True Day & Night: Yes (ICR)

- IR Distance (The number of LEDs, IR wavelength): 15m (3ea)

- Intelligent Video Analytic:

Object Metadata: Human, Vehicle, Face

IDLA: Object Detection, Intrusion, Loitering, Line Crossing, Face Detection

ETC: Video Motion Detection, Active Tampering Alarm

Audio Specification

- Audio Compression Algorithm: OPUS, ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law

- Audio Input / Output: Line-in 1ea / Line-out 1ea

- Audio Impedance:

    - Max output level : 1Vrms

    - Input Impedance : 3.6k Ohm

    - Output Impedance : 174.25 Ohm

- Two-way Audio Communication: Yes

- Pre-recorded Voice Alert(Audio Alarm): Yes

Network Specification

- Port: RJ-45 10/100 Base-T 1 port

- Network Protocols: DirectIP 2.0 Protocol, IPv4, IPv6, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP,

RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP, SNMPv2

- Streaming Mode: Quadruple Streaming

Security Specification

- Digest authentication, Password Protection, User access log, IP Filtering, IEEE802.1x(EAP-TLS), Chained Fingerprint, SSL Encryption, HTTPS Encryption

- Maximum User Access: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

Alarm and Event Specification

- Alarm Input / Output: 1 / 1

  - Alarm Input: TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

  - Alarm Output: 1 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Audio detection, Tampering, Object Detection, Intrusion, Loitering, Line Crossing, Face Detection

- Event Notification: Remote S/W, Email (with Image)

Environmental Specification

- Operating Temperature: -40°C ~ +55°C (-40°F ~ +131°F)

*Starting up at above -20°C (-4°F)

- Operating Humidity: 0% to 90% non-condensing

- Vandal-proof Enclosure: IK10

- Outdoor Ready: IP67, NEMA4X

Electrical Specification

- Power Source: PoE, IEEE 802.3at(Class 4)

- Power Consumption:

    - PoE, IEEE 802.3af Class 3, 25W [TBD]

- Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (Ø x H): Ø258 x 112mm(Ø10.2” x 4.4”)

- Unit Weight: 2.8kg (6.2lb)

Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0.0 | TS Team | Aug. 25. 2025 | Initial Release |