DC-Y8D11WRA

Architectural and Engineering Specifications

Version 1.0

(Feb. 25, 2026)

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13493, Korea
Tel: 	+82 31 723 5400
Fax: 	+82 31 723 5100

General

Product Description

DC-Y8D11WRA is a Network Camera (IP Camera) designed and manufactured by IDIS. This camera is a fisheye camera which provides 12MP (3520 x 3520) resolution at 30ips (images per second) with H.265/H.264/MJPEG compression. The camera is equipped with Fixed-focal lens, True Day/Night, PoE (IEEE 802.3at Class 4), Audio I/O + Built in Mic, Alarm I/O, microSD/SDHC/SDXC backup (Smart Failover up to 512GB), Vandal-proof/Weather-proof and IP67 rated.

General Specification

- The IP camera shall be equipped with 12 Megapixel 1/1.6” CMOS Sensor.

- The IP camera shall be equipped with 1.7mm Fixed-focal lens, F2.0.

- The IP camera shall be a true day/night camera with a mechanical filter for low light performance.
The filter can be switched remotely, or automatically via a light level sensor or contact input (ICR).

- The IP camera shall have wide dynamic range compensation (True WDR) for improved video quality in high-contrast situations (120dB).

- The IP camera shall utilize configurable 3DNR (Dynamic Noise Reduction) technology to reduce the bitrate and storage requirements by removing noise artifacts.

- The IP camera shall be equipped with 10/100/1000 Base-T, auto-sensing, half/full duplex, RJ-45 Ethernet connection.

- The IP camera shall be vandal-resistant with an IK10 rating and compliant with IP67 for dust and water protection.

- The IP camera shall support industry standard Power over Ethernet (PoE) IEEE 802.3at, Class 4 to supply power to the camera over the network and 12VDC input.

- The IP camera shall have on board microSD/SDHC/SDXC card backup storage slot as a safeguard against data loss during network interruptions.

- Using IDIS NLTSrec(Non-Linear Time Shifting recording) technology, the IP camera can store the recording data to the internal recording memory buffer (60MB) in camera if there is a delay in data transmission due to the instantaneous load of the recorder or network, and then transmits the stored data to IDIS recorder safely.

- The IP camera shall deliver maximum video resolution of 3520 x 3520 at rates up to 30ips (images per second).

- The IP camera shall provide direct network connection using H.265, H.264 and M-JPEG compression.

- The IP camera shall support Quadruple Streams.

- The IP camera shall be compliant with DirectIP 2.0 and support ONVIF Profiles S, T, M, and G (subject to confirmation).

- The IP camera shall be equipped with embedded web server which works independently using a Web Browser with ActiveX plug-in.

- The IP camera shall provide advanced security features, including digest authentication, password protection, user access logs, IP filtering, IEEE 802.1X (EAP-TLS), chained fingerprint, SSL encryption, and HTTPS encryption.

- The IP camera shall have network bandwidth limitation and MAT features for more efficient use of network bandwidth.

- The IP camera shall have Easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall support Intelligent Video Analytics (VA), including motion detection, alarm input, audio detection, tampering detection, object detection, intrusion detection, loitering detection, and line crossing detection, along with attribute analysis for human and vehicle classification.

- The IP camera shall provide a fisheye imaging system with dewarping capabilities and SmartUX control for dynamic and seamless panoramic viewing.

- The IP camera shall support panoramic intelligent object tracking

- The IP camera shall be equipped with built-in IR LEDs with an illumination distance of up to 20 meters and support zone adaptive IR (4 zones).

- The IP camera shall support dynamic power balancing for efficient power utilization.

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall support camera can be linked to IDIS software solution such ad IDIS Center and IDIS Solution Suite, or 3rd party solution while it is being connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/1.6” CMOS

- Maximum Resolution: 3520 x 3520

- Scanning Mode: Progressive Scan

- Lens Type: Fixed-focal (f=1.7mm, F2.0)

- Iris Control: Fixed Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - 180º(H), 180º(V), 180(D)

- Minimum Illumination:

  - Color: 0.13 lux @ F2.0

  - B/W: 0 lux (IR LED ON)

- S/N Ratio: 45dB

- Maximum Frame Rate:

  - 30ips : 3520 x 3520

  - 15ips : 3520 x 3520 (WDR)

- Video Resolution: 3520x3520, 3520x1760, 2640x2640, 1760x1760, 880x880, 880x440

- Video Compression: H.265, H.264 and M-JPEG

- Video Compression Level: Basic, Standard, High, Very High

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB (Ture WDR)

- True Day & Night: Yes (ICR)

- IR Distance (The number of LEDs, IR wavelength): 20m (65.6ft) / 4ea & Zone Adaptive IR

- Intelligent Video Analytic:

- A-cut Support

- Object Metadata: Human, Vehicle

- Object Attribute: Human [Gender, Age, Bag, Hat, Colors], Vehicle [Car, Truck, Bus, Bike(Bicycle, Motorcycle), ETC, Colors]

- IDLA: Object Detection, Intrusion, Loitering, Line Crossing

- ETC: Video Motion Detection, Active Tampering Alarm

Audio Specification

- Audio Compression Algorithm: Opus, ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law

- Audio Input / Output: Line-in 1ea + Internal Mic 1ea / Line-out 1ea

- Two-way Audio Communication: Yes

- Pre-recorded Voice Alert: Yes

Network Specification

- Port: RJ-45 10/100/1000 Base-T 1 port

- Network Protocols: Digest authentication, Password Protection, User access log, IP Filtering, IEEE802.1x(EAP-TLS), Chained Fingerprint, SSL Encryption, HTTPS Encryption

- Streaming Mode: Quadruple streaming

Security Specification

- Digest authentication, Password Protection, User access log, IP Filtering, IEEE802.1x(EAP-TLS), Chained Fingerprint, SSL Encryption, HTTPS Encryption

- Maximum User Access: 10 (Live), 1 (Recording), 1 (Search), 2 (Admin)

Alarm and Event Specification

- Alarm Input / Output: 1 / 1

  - Alarm Input: 1 TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

  - Alarm Output: 1 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Audio detection, Tampering, Object Detection, Intrusion, Loitering, Line Crossing

- Event Notification: Remote S/W, Email (with Image)

Environmental Specification

- Operating Temperature: -40°C ~ 55°C (-40°F ~ 131°F)

- Operating Humidity: 0% to 90% non-condensing

- Vandal-proof Enclosure: IK10

- Outdoor Ready: IP67

Electrical Specification

- Power Source: 12VDC, PoE(IEEE 802.3at class 4)

- Power Consumption:

  - 12V=, 1.9A, 22.8W

  - PoE (IEEE 802.3at Class 4), 24W

- Regulatory Approvals: UL, FCC, CE, KC

Mechanical Specification

- Dimensions (Ø x H): Ø157.5mm x 73mm (Ø6.2" x 2.87")

- Unit Weight: 1.1 kg (2.42 lb)

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0 | TS Team | June 5, 2019 | Initial Release |
| 1.1 | TS Team | Mar. 23, 2021 | Spec Update |
| 1.2 | TS Team | Aug. 31, 2022 | Spec Update |