DC-S6683WRA

Architectural and Engineering Specifications

Version 1.1.0

PART 1: PLEASE REFER TO ATTACHED DOCUMENTS - OVERVIEW & FORMAT SAMPLES

PART 2: PRODUCTS

Division 28 – Electric Safety and Security

Level 1 - 28.20.00 – Video Surveillance

Level 2 - 28.21.00 – Surveillance Cameras

Level 3 - 28.21.13 – IP Cameras

Manufacturer

- IDIS Co., Ltd.
IDIS Tower, 344 Pangyo-ro, Bundang-gu
Seongnam-si, Gyeonggi-do, 13494, Republic of Korea
Tel: 	+82 31 723 5028
Fax: 	+82 31 723 5100

General

Product Description

The DC-S6683WRA is a 6MP PTZ camera designed for professional surveillance. It sup	s H.264, H.265, and MJPEG compression formats, combined with IDIS Intelligent Codec for efficient bandwidth and storage usage. The camera has a 40x AF optical zoom lens (6.5–260mm, F1.69–F4.92) with P-Iris control. It is rated IK10 vandal-resistant and IP66/NEMA4X weatherproof. Long-range IR LED illumination covers up to 400m, and True WDR (120dB) improves visibility under varying lighting conditions. The A-Cut feature is an AI-based technology from IDIS that extracts and indexes objects or people in real-time video for search and tracking. Smart Failover allows local recording on microSD/SDHC/SDXC cards up to 512GB during network outages. The camera supports ONVIF and can be integrated with IDIS DirectIP NVRs. Additional features include Alarm In/Out and power options via 24VAC or PoE (IEEE 802.3bt Class 8).

General Specification

- The IP camera shall be equipped with a 6-megapixel 1/1.8" CMOS sensor (3328 x 1872).

- The IP camera shall be equipped with a 6.5–260mm motorized AF optical zoom lens, F1.69–F4.92, with P-Iris control.

- The IP camera shall be a true day/night camera with a mechanical IR cut filter (ICR) for improved low-light performance, switchable automatically via light sensor or remotely.

- The IP camera shall support True Wide Dynamic Range (WDR) for improved image clarity in high-contrast scenes (120 dB).

- The IP camera shall be equipped with long-range IR LED illumination up to 400m (1,312 ft).

- The IP camera shall support 2DNR and 3DNR (Dynamic Noise Reduction) to reduce image noise and optimize storage and bandwidth usage.

- The camera shall be enclosed in a dome housing with vandal resistance (IK10) and IP66/NEMA4X weatherproof protection.

- The camera shall have a 10/100Base-T RJ-45 Ethernet port with auto-sensing and full/half-duplex mode.

- The camera shall support Power over Ethernet (PoE) IEEE 802.3bt Class 8 and 24VAC power input.

- The camera shall include a CVBS video output for local setup and testing.

- The IP camera shall be equipped with an on-board microSD/SDHC/SDXC card slot, supporting up to 512GB for edge recording.

- The camera shall support Smart Failover to prevent data loss during short network interruptions or recorder overload.

- The camera shall support a maximum resolution of 3328 x 1872 at up to 30 ips (images per second).

- The IP camera shall provide network connection using H.265, H.264, and MJPEG compression.

- The IP camera shall support quadruple streams in DirectIP 2.0 protocol mode.

- The IP camera shall conform to the ONVIF Profile S/T/M.

- The IP camera shall be equipped with an embedded web server (IDIS Web) which works independently using a web browser with ActiveX plug-in.

- The IP camera shall include security features such as: Digest authentication, Password protection, Brute force delay protection, User access log, IP filtering, IEEE802.1x(EAP-TLS), SSL Encryption, HTTPS Encryption.

- The IP camera shall support bandwidth limitation and MAT for optimized network usage.

- The IP camera shall have easy network access via UPnP (Universal Plug and Play) function and embedded mDNS (Multicast DNS) protocol.

- The IP camera shall have Digital Image Stabilization (EIS) to reduce the effect of camera shake or vibration.

- The IP camera shall have Intelligent Video Analytics, including:

- A-Cut Support

- Object Metadata: Human, Vehicle, Face

- Object Attributes: Human [Gender, Age, Bag, Hat, Colors], Vehicle [Car, Truck, Bus, Bike (Bicycle, Motorcycle), ETC, Colors], Face [Gender, Age, Glasses, Hat, Mask]

- IDLA: Object Detection, Intrusion, Loitering, Line Crossing, Face Detection

- IDLA Pro: Crowd Detection, Abandoned Object Detection, Removed Object Detection, Fall Detection

- ETC: Video Motion Detection, Active Tampering Alarm

Protocol Specification: DirectIP 2.0

- The IP camera shall have DirectIP 2.0 mode.

- DirectIP 2.0 protocol shall provide easy connection to DirectIP NVR for automatic discovery and video streaming configuration.

- DirectIP 2.0 shall provide the compatibility with IDIS Solution Suite VMS or ONVIF for third-party software solutions.

- DirectIP 2.0 shall allow the camera to be linked to IDIS software solutions such as IDIS Center and IDIS Solution Suite, or to third-party solutions, while connected to a DirectIP NVR.

- DirectIP 2.0 camera shall be compatible with DirectIP 1.0 NVR as well as DirectIP 2.0 NVR.

- DirectIP 2.0 camera shall be unavailable for No-password login when connecting to DirectIP 2.0 NVR and IDIS Software Solutions.

- DirectIP 2.0 protocol shall provide Quadruple streams.

- DirectIP 2.0 protocol shall support H.264 and H.265 and MJPEG compression.

Technical Specification

Video Specification

- Image Sensor: 1/1.8” CMOS

- Maximum Resolution: 3328 x 1872

- Scanning Mode: Progressive Scan

- Lens Type: AF Zoom Lens

  - f= 6.5mm ~ 260mm

  - F1.69 ~ F4.92

- Iris Control: P-Iris

- Angular Field of View (H: Horizontal, V: Vertical, D:Diagonal):

  - Wide: 64.6º(H), 38.4º(V), 72.2º(D)

  - Tele: 1.79º(H), 1.00º(V), 2.08(D)

  - Pan Range, Speed: 360° (Endless), 0.02 ~ 240°/sec (preset 240°/sec)

  - Tilt Range, Speed: 100° (-10~90°), 0.02 ~ 240°/sec (preset 240°/sec)

  - Zoom Range: x40

- Minimum Illumination:

- Color : 0.13 lux @ F1.69

- B/W : 0 lux (IR LED On)

- S/N Ratio: more than 45dB

- Maximum Frame Rate: 30ips : 3328 x 1872(WDR)

- Video Resolution: 3328 x 1872, 1920x1080, 1280x720, 640x360

- Video Compression: H.265, H.264 (MP), M-JPEG

- Video Compression Level: 4 levels: Basic, Standard, High, Very High

- Video Bitrate Control: H.264 - CBR / VBR, H.265 - CBR / VBR

- Intelligent Codec: Off (Default) / On

- Motion Adaptive Transmission (MAT): Off (Default) / On

- Multi-Video Streaming: Quadruple streams

- Dynamic Range: 120dB (True WDR)

- Electronic Shutter Speed: Auto / Manual (1/30 ~ 1/10000), Anti-Flicker, Slow Shutter (1/7.5, 1/15)

- True Day & Night: Yes (ICR)

- IR Distance (number of LEDs): 400 m / 1,312.3ft (2ea)

- Image Setting: Configurable Exposure, White Balance, Self-Adjusting Video Mode(Sharpness, Contrast, Colors, Brightness Control)

- Mirroring / Pivot: Horizontal / Vertical

- Image Stabilizer: Yes (EIS)

- Privacy Masking: 32 Zones

  - Intelligent Video Analytic: A-cut Support

  - Object Metadata: Human, Vehicle, Face

  - Object Attribute: Human[Gender, Age, Bag, Hat, Colors], Vehicle[Car, Truck, Bus, Bike(Bicycle, Motorcycle), ETC, Colors], Face[Gender, Age, Glasses, Hat, Mask]

  - IDLA: Object Detection, Intrusion, Loitering, Line Crossing, Face Detection

  - IDLA Pro: Crowd Detection, Abandoned Object Detection, Removed Object Detection, Fall Detection

  - ETC: Video Motion Detection, Active Tampering Alarm

- Analog Video Output: 1 Terminal Block

- Advanced PTZ options: 256 presets; Auto Tracking: Yes

- De-fog : Yes

Audio Specification

- Audio Compression Algorithm: Opus, ADPCM 16K, G.726, G.711 u-Law, G.711 a-Law

- Audio Input / Output: Line-in 1ea / Line-out 1ea

- Audio Impedance:

  - Max output level : 1Vrms

  - Input impedance (1Vpp 1KHz) : 3.79kΩ

  - Output Impedance (1Khz) : 306.9Ω

- Two-way Audio Communication: Yes

Network Specification

- Port: RJ-45, 10/100Base-T, 1 port

- Network Protocols: DirectIP 2.0 Protocol, IPv4, IPv6, RTP/RTSP/TCP, RTP/RTSP/HTTP/TCP, RTP/UDP RTSP/TCP, HTTP, HTTPS, FTP, SNTP, SMTP, FEN, mDNS, uPNP, SNMPv2

- Security: Digest authentication, Password Protection, User access log, IP Filtering, IEEE802.1x(EAP-TLS), Chained Fingerprint, SSL Encryption, HTTPS Encryption

- Remote Access Client: DirectIP NVR Connection, IDIS Web, IDIS Mobile, IDIS Solution Suite

- Recording Session Buffer (NLTSrec): Up to 60MB

- Edge Storage: micro SD/SDHC/SDXC, Smart Failover (Up to 512GB)

Security Specification

- FIPS 140-3 Level 3, Digest authentication, Password Protection, Brute force delay protection, User access log, IP Filtering, IEEE802.1x(EAP-TLS), Chained Fingerprint, SSL Encryption, HTTPS Encryption

- Maximum Concurrent Sessions: 10 sessions shared across Admin, Watch, and Search, with a maximum of 1 Search session.

Alarm and Event Specification

- Alarm Input / Output: 8 / 2

  - Alarm Input: 8 TTL, NC/NO Programmable, 4.3V(NC) or 0.3V(NO) threshold, 5V DC

  - Mechanical or electrical switches can be wired to the Alarm-In and GND connectors. The maximum voltage should not exceed 5V.

  - Alarm Output: 2 TTL open collector, 30mA @ 5 VDC

- Trigger Events: Motion Detection, Alarm in, Audio detection, Tampering, Object Detection, Intrusion, Loitering, Line Crossing, Face Detection, Crowd detection, Abandoned object detection, Removed object detection, Fall detection

- Event Notification: Remote S/W, Email (with Image)

- Audio Alarm: Yes

Environmental Specification

- Operating Temperature:

  - IRLED OFF : -40°C ~ +55°C (-40°F ~ +131°F)

  - IRLED ON :  -40°C ~ +40°C (-40°F ~ +104°F)

*Starting up at above -20°C (-4°F)

- Operating Humidity: 0% to 90%

- Vandal-proof Enclosure: IK10

- Outdoor Ready: IP66, NEMA4X, Heater

Electrical Specification

- Power Source:24VAC, PoE(IEEE 802.3bt, Class 8)

- Power Consumption:

  - HEATER, IR OFF

- 24V~, 2.0A, 36.4W, 50/60Hz

- PoE, IEEE 802.3bt(Class 8), 34.8W

B.  HEATER, IR ON

- 24V~, 3.0A, 55.9W, 50/60Hz

- PoE, IEEE 802.3bt(Class 8), 55.7W

3. Regulatory Approvals: FCC, CE, KC

Mechanical Specification

- Dimensions (Ø x H): Ø242mm x 401.8mm (Ø9.52" x 15.82")

- Unit Weight: 7.46kg (16.44 lbs)

Software Specification

- Supported Language: English, French, German, Italian, Spanish, Dutch, Polish, Portuguese, Hungarian, Czech, Russian, Danish, Swedish, Finnish, Turkish, Croatian, Korean, Japanese, Chinese-PRC, Chinese-Taiwan

- Version History

| Version | Writer | Revision Date | Remarks |
| --- | --- | --- | --- |
| 1.0.0 | GT Team | Sep. 08, 2025 | Initial Release |
| 1.1.0 | GT Team | Oct. 14, 2026 | Updated product specifications and corrected document errors |